	[image: image1.png]



	SI M51 03 09
Body Equipment
	May 2009
Technical Service


[image: image2.png]



SUBJECT
Noises from the Closed Convertible Top 

MODEL
R57 (Cooper and Cooper S Convertible) 

SITUATION
When driving over rough surfaces with the convertible top in the closed position, a grating or squeaking noise can be heard from the area of the convertible top seals. If the convertible top is opened, or if the windows are lowered slightly, the noise is no longer present. 

CAUSE
A slip-stick effect between the glass and the mating seals 

CORRECTION
Isolate or treat the affected seals. 

PROCEDURE
Refer to the attached procedure, SI M51 03 09 Procedure.pdf , for complete details. 

PARTS INFORMATION
	Part Number
	Description
	Quantity

	61 13 6 920 760
	Felt tape
	1

	83 23 0 309 627
	Carbaflow
	1


WARRANTY INFORMATION
Covered under the terms of the MINI New Vehicle Limited Warranty. 

	Defect Code:
	54 34 25 40 00
	

	
	
	

	Labor Operation:
	Labor Allowance:
	Description:

	54 34 337
	Refer to KSD
	R&I front seal, left or right

	
	
	

	+54 34 843
	Refer to KSD
	R&I rear seal, left or right

	
	
	

	54 99 000
	2 FRU
	Work time to apply isolation materials


	*Main Work - use this labor operation number when this is the only repair being performed, or if this is the main repair when performed along with other repairs at the same time. If this is not the main repair, refer to KSD for the associated (+) labor operation code.


	Sublet Code 4**
	$3.00
	** Reimbursement of up to the amount listed may be charged to sublet code 4 for the Felt tape and Carbaflow if necessary to perform this repair. Claiming this part number outside of sublet code 4 will result in a delayed or denied claim payment.


ATTACHMENTS
view PDF attachment M510309Procedure.


	[ MINI, a division of BMW of North America, LLC
Copyright © 2009 BMW of North America, LLC ] 


	[image: image3.png]



	SI M51 02 09
Body Equipment
	May 2009
Technical Service


[image: image4.png]



SUBJECT
Noises from the Open Convertible Top 

MODEL
R57 (Cooper and Cooper S Convertible) 

SITUATION
When driving over rough surfaces with the convertible top open, a rattling or grinding noise can be heard from the rear seat/convertible top area. If the convertible top is closed, the noise is no longer present. 

CAUSE
Hard contact between areas of the convertible top that have some movement when in the open position, caused by tight tolerances or incorrect adjustments 

CORRECTION
Adjust or isolate the part in contact. 

PROCEDURE
Refer to the attached procedure, SI M51 02 09 Procedure , for complete details. 

PARTS INFORMATION
	Part Number
	Description
	Quantity

	61 13 6 920 760
	Felt tape
	1 – if necessary

	83 19 9 407 798
	Teflon tape
	1 – if necessary


WARRANTY INFORMATION
Covered under the terms of the MINI New Vehicle Limited Warranty. 

	Defect Code:
	54 34 25 40 00
	

	
	
	

	Labor Operation:
	Labor Allowance:
	Description:

	54 99 000 
	4 FRU
	Work time to adjust and isolate the areas noted above.


	*Main Work - use this labor operation number when this is the only repair being performed, or if this is the main repair when performed along with other repairs at the same time. If this is not the main repair, refer to KSD for the associated (+) labor operation code.


	Sublet Code 4**
	$3.00
	** Reimbursement of up to the amount listed may be charged to sublet code 4 for the Felt and Teflon tape if necessary to perform this repair. Claiming this part number outside of sublet code 4 will result in a delayed or denied claim payment.


ATTACHMENTS
view PDF attachment M510209Procedure.


	[ MINI, a division of BMW of North America, LLC
Copyright © 2009 BMW of North America, LLC ] 


