[image: image1.jpg]



RISS RACING O2 SIM DIY/ADAPTION SEQUENCE (SUPPLEMENT)

Please read DIY for full instruction list.  These are minor changes that need to be made when installing the SIM.

1. Make sure connections are ON and tight.  If need be, solder on system.

2. Ground o2 sim wire (Black near o2 sim/Brown near male/female connectors) to the BODY of the car, preferably near the battery (red) wire.

3. Use standard/Mini OBDII tool and clear codes.  Higher end OBDII scan tools ($100-$150) should also be able to show shadow/pending codes.  



ADAPTION SEQUENCE:
Courtesy of JamesHunt of NorthAmericanMotoring.com

1. Clear all codes and start vehicle.  

2. Drive vehicle and do not exceed 60MPH.  Use wide-open throttle in 3rd and 4th gear only (full throttle to max 60mph then lift and coast down in gear), try to keep speed in 30 to 50mph range and never at a constant mph for a distance of 25 miles.

3. Drive on Highway at speeds of 60-75mph using wide open throttle in 4th and 5th gear only(full throttle to 75 mph then lift while keeping in gear) and never at a constant mph for a distance of 25 miles.

4. Drive vehicle from 25-35mph not at a constant RPM.  CAR SHOULD BE ADAPTED!

**for some drivers, they have seen the CEL turn on and then disappear while car finalizes adapting.   Check for shadow/pending codes.

