ok, here's the pics, as promised ( I say I'll give pics and i deliver :P ), please NOTE, the pics make the leds look lighter blue than they really are, they are 470nm, so not *deep* blue, but a midway between light blue and deep, they are harder to see in the day light too, but still possible to see... it took me a few hours (tho i did one "Group" at a time, i.e. tach and speedo, auto ac, switches, etc.. separately.. the LEDs on the bottom of hte switch panel as well as the ones in the seat heater controls are Right Angle LEDs, i had to improvise and take normal ones and put them on their sides.. Behind the LCD on the Odometer and the clock have an orange plastic diffuser, while leaving that in will still show blue, it diffuses too much, and i ended up using the waxy paper from a Walgreen Prescription bag, works fine, and avoids the spotlight affect from the LEDs., i DID end up using small pieces of the orange plastic to dim down the heated seat switches and the flat tired reset LEDs, they're way too bright otherwise.

here are the LED *sizes*
the majority of the LEDs are, PLCC size, digikey part number: 160-1229-1-ND
the LEDs under the menu button on the NAV, as well as on either end of the LED gas gauge are 805s, digikey part num: 160-1579-1-ND
the LEDs in the cruise control buttons are, 1206s, part number: 160-1401-1-ND
the LED in the Flat Tire reset is a T1 sized blue LED.

