MINI Cooper Services Explained


Mini Coopers have 3 types of service: 
1. Oil Change 

2. Inspection I 

3. Inspection II
 Your Service interval display located in the center of the speedometer will notify you of which service type you need. Each time the vehicle is started the display will show the mileage until the next service is needed. 
 

If the display is flashing and the mileage is displayed as a negative you have gone past the time of service and are overdue. 
The services come in this order, 
1. Oil change 

2. Inspection I 

3. Oil Change 

4. Inspection II 

5. repeat 

Inspection I is also known as a Minor service and Inspection II is also known as a Major Service.
Inspection II includes most of the tasks from Inspection I with additional Inspection II tasks. 
 

OIL SERVICE:
· Oil and Filter  

· Reset service interval display  

· Interior Cabin Filter  

· Check brake pads  

· Check brake rotors  

· Clean brake pad contact points  

· Grease wheel centering hubs  

· Check parking brake 



INSPECTION I:
· Oil and Filter  

· Check Transmission Fluid for leaks  

· Change CVT (Automatic Transmission) fluid  

· Check CV joint boots for damage or leaks  

· Visually check fuel tank, fuel lines and connections for leaks  

· Check condition, position and mounting of exhaust system.  Adjust if necessary.  

· Check struts and shock absorbers for leaks  

· Check front control arm bushings for damage or wear  

· Check power steering system for leaks.  Check fluid level and add if necessary.  

· Check steering rack and tie rods for tightness. 

· Check condition of steering linkage and steering shaft joints. 

Check overall thickness of front and rear brake pads.
 If replacement is necessary: 
·   - Examine Brake disc surface   

·   - Clean brake pad contact points in calipers   

·   - Grease wheel centering hubs   

·   - Check thickness of parking brake linings only when replacing rear brake pads   

·   - Check operation of parking brake and adjust as necessary 
 Check brake system connections and lines for leaks, damage and incorrect positioning  

· Check parking brake actuator.  Adjust if necessary.  

· Check all tire pressures (including spare if equipped) and correct if necessary.  

· Check condition of tire tread surface, tread wear pattern.  Align if necessary.  

· Inspect entire body according to terms of rust perforation limited warranty. (Must be performed every 2 years)
· Read out on-board diagnostic (OBDII) fault codes
· Check engine coolant system / heater hose connections for leaks.
· Check coolant level and antifreeze protection level.  Add if necessary
· Check windshield washer fluid level and antifreeze protection.  Add if necessary.
· Check air conditioner for operation.
· Replace ventilation micro-filter. (Reduce replacement interval in dusty conditions.)
· Reset service interval display. 

Check battery state of charge

Check operation of: 
·   - Headlights, fog lights, parking lights, back-up lights, turn signals,   

·   - Emergency flashers, stop lights, license plate lights, interior lights,   

·   - Glove compartment light, make-up mirror light, flashlight,   

·   - Engine compartment light, trunk light. 
· Check instrument panel lights  

· Check warning and indicator lights, check control  

· Check operation of horn, headlight dimmer and flasher switch  

· Check wipers and windshield washer system  

· Check aim of washer jets and adjust if necessary  

· Visually examine all airbag units for torn covers, obvious damage or attachment of decals, decorations, or accessories 

· Check central locking system  

· Check operation and condition of door, engine hood and rear hatch latches  

· Check heater, air-conditioner blower and rear defogger operation  

· Check operation of rear view mirrors 


Check:
· Braking performance 

· Steering 

· Heating 

· Air-conditioner operation 

Check manual transmission and clutch or automatic transmission operation

INSPECTION II:
· Air filter Element  

· Serpentine Belt - 60,000 miles for Cooper S  

· Brake Fluid - Every 2 years (dealer recommended)  

· Coolant - Every 4 years  

· Oxygen Sensor - Replace at 100,000 miles  

· Spark Plugs - Replace at 100,000 miles
3

